Town of Nags Head

Permeable Pavement Operation and Maintenance Agreement

I will keep a maintenance record on this BMP.  This maintenance record will be kept in a log in a known set location.  Any deficient BMP elements noted in the inspection will be corrected, repaired or replaced immediately.  These deficiencies can affect the integrity of structures, safety of the public, and the removal efficiency of the BMP.

Important operation and maintenance procedures:

- Stable groundcover will be maintained in the drainage area to reduce the sediment load to the permeable pavement

- The area around the perimeter of the permeable pavement will be stabilized and mowed, with clippings removed.

- Any weeds that grow in the permeable pavement will be sprayed with pesticide immediately.  Weeds will not be pulled, since this could damage the fill media.

- Once every two years, the permeable pavement surface will be vacuum swept.

- At no time shall wet sweeping (moistening followed by sweeping) be allowed as a means of maintenance.

- There shall be no repair or treatment of permeable pavement surfaces with other types of pavement surfaces.  All repairs to permeable pavement surfaces must be accomplished utilizing permeable pavement which meets the original pavement specifications.

- Concentrated runoff from roof drains, piping, swales or other point sources, directly onto the permeable pavement surface shall not be allowed.  These areas must be diverted away from the permeable pavement.

The permeable pavement shall be inspected once every two years by a licensed engineer.  Records of inspections, operation and maintenance shall be provided to the Town of Nags Head Planning and Development Department within two weeks of performance.

Inspection activities shall be performed as follows.  Any problems found shall be repaired immediately.

| BMP Element: | Potential Problem: | How to remediate the problem: |
| --- | --- | --- |
| The perimeter of the permeable pavement | Areas of bare soil and/or erosive gullies have formed. | In the event that rutting or failure of the groundcover occurs, the eroded area shall be repaired immediately and a permanent groundcover re-established. Appropriate temporary erosion control measures, (such as silt fence), shall be installed in the affected area during the establishment of a permanent groundcover, and any impacted area of permeable pavement is to be cleaned via vacuum sweeping. Regrade the soil if necessary to remove the gully, and then plant a ground cover and water until it is established. Proved lime and a one-time fertilizer application. |
|  | Vegetation is too short or too long | Maintain vegetation at a height of 3 to 6 inches (remove clippings). |
| The surface of the permeable pavement | Trash/debris is present. | Remove the trash/debris. |
|  | Rutting/uneven settlement | This indicates inadequate compaction of the pavement base/sub-base. If rutting or even settlement on the order of ½ or greater occurs, permeable pavement shall be removed and the base, sub-base re-compacted, smoothed, and the permeable pavement re-installed. Base and sub-base compaction shall be monitored by a N.C. licensed engineer to ensure the infiltration capacity of the base and sub-base material are not compromised by the compaction and smoothing process. |
|  | Weeds are growing on the surface of the permeable pavement. | Do not pull the weeds (may pull out media as well). Spray them with pesticide. |
|  | Sediment is present on the surface. | Vacuum sweep the pavement. |
|  | The structure is deteriorating or damaged. | Consult an appropriate professional. Damaged areas of the pavement shall be removed and repaired. |
|  | The pavement does not dewater between storms. | Vacuum sweep the pavement. If the pavement still does not dewater, consult a professional. Permanently clogged pavement shall be removed and repaired. No sealing of the porous pavement shall be permitted. |

I acknowledge and agree by my signature below that I am responsible for the performance of the maintenance procedures listed above.  I agree to notify the Town of Nags Head of any problems with the system or prior to any changes to the system or responsible party.

Property Address: _____________________________________________________

Print Name: ___________________________________________________________

Title: _________________________________________________________________

Address: ______________________________________________________________

Phone: ________________________________________________________________

Signature: ______________________________________________________________

Date: _____________________________

I, ______________________________________, a Notary Public for the State of ___________________, County of ____________________, do hereby certify that ____________________________________ personally appeared before me this _______ day of _______________, _______, and acknowledge the due execution of the forgoing permeable pavement maintenance requirements.  Witness my hand and official seal,

My commission expires ________________________________________